| ASSIGNMENT |  |
| --- | --- |
| Course Code | ECC201A |
| Course Name | Signals and Systems |
| Programme | B.Tech |
| Department | CSE |
| Faculty | FET |

| Name of the Student | Satyajit Ghana |
| --- | --- |
| Reg. No | 17ETCS002159 |
| Semester/Year | 04/2019 |
| Course Leader/s | Ms Prafulla Kumari |

| # Declaration Sheet |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Student Name | Satyajit Ghana |  |  |  |  |  |  |  |
| Reg. No | 17ETCS002159 |  |  |  |  |  |  |  |
| Programme | B.Tech |  |  |  |  | Semester/Year | 04/2019 |  |
| Course Code | ECC201A |  |  |  |  |  |  |  |
| Course Title | Signals and Systems |  |  |  |  |  |  |  |
| Course Date |  |  | to |  |  |  |  |  |
| Course Leader | Ms. Prafulla Kumari |  |  |  |  |  |  |  |
| Declaration The assignment submitted herewith is a result of my own investigations and that I have conformed to the guidelines against plagiarism as laid out in the Student Handbook. All sections of the text and results, which have been obtained from other sources, are fully referenced. I understand that cheating and plagiarism constitute a breach of University regulations and will be dealt with accordingly. |  |  |  |  |  |  |  |  |
| Signature of the Student |  |  |  |  |  |  | Date |  |
| Submission date stamp (by Examination & Assessment Section) |  |  |  |  |  |  |  |  |
| Signature of the Course Leader and date |  |  |  | Signature of the Reviewer and date |  |  |  |  |

# Contents

____________________________________________________________________________

Declaration Sheet	ii

Contents	iii

List of Tables	v

List of Figures	vi

List of Symbols	vii

Question No. 1	8

1.1 Overview:	8

1.2 Solution to the question:	8

1.3 Discussions /Suggestions/Views/Recommendations	8

1.4 Conclusions	8

Question No. 2	9

2.1 Overview:	9

2.2 Solution to the question:	9

2.3 Discussions /Suggestions/Views/Recommendations	9

2.4 Conclusions	9

Question No. 3	10

3.1 Overview:	10

3.2 Solution to the question:	10

3.3 Discussions /Suggestions/Views/Recommendations	10

3.4 Conclusions	10

Question No. 4	11

4.1 Overview:	11

4.2 Solution to the question:	11

4.3 Discussions /Suggestions/Views/Recommendations	11

4.4 Conclusions	11

Question No. 5	12

5.1 Overview:	12

5.2 Solution to the question:	12

5.3 Discussions /Suggestions/Views/Recommendations	12

5.4 Conclusions	12

# List of Figures

____________________________________________________________________________

| Figure No. | Title of the figure | Pg.No. |
| --- | --- | --- |
| Figure 1.1 | Title of the figure | 13 |
| Figure 1.2 | Title of the figure | 15 |
| Figure 2.1 | Title of the figure | 19 |

# Question No. 1

Solution to Question No. 1 Part A:

## A 1.1 Introduction:

Overview to the question (students are expected to give a brief introduction to the context on which the question is set, applications, limitations, new developments happening and students own views on the question and the paragraph should not exceed 200 words and references should be cited and it should be authored by the students means to say students should not be borrowing sentences as they are from any referred literature)

## A 1.2 Role of difference equation in the analysis of time series:

Students are expected to provide the solution to the question considering the points mentioned in the marking scheme of the assignment question

## A 1.3 Conclusion:

Students are expected to discuss the solutions obtained in section 1.2 and present their views/suggestions/recommendations (not to exceed 150 words)

# Question No. 2

Solution to Question No. 1 Part B:

Given:

ASCII String: VN

Transmission rate: 1Mbps

Amplitude: 3.5V

Frequency for bit 1: 10Mhz

Frequency for bit 0: 5Mhz

## B 1.1 Determine the NRZ encoding of the text string

The given 2-character ASCII string is first converted to its binary representation,

V = 86 = 1010110

N = 78 = 1001110

a = 3.5.*[1 -1 1 -1 1 1 -1 1 -1 -1 1 1 1 -1 -1];

x = [0:length(a)-1];

stairs(x, a, 'LineWidth', 2);

ax1 = gca;

set(ax1, 'FontSize', 14);

xticks(0:length(a)-1);

xticklabels({'       1', '       0', '       1', '       0', '       1', '       1', '       0',...

'       1', '       0', '       0', '       1', '       1', '       1', '       0'});

yticks([-3.5 0 3.5])

axis([0 length(a)-1 -5 5])

grid on;

xlabel('$t \rightarrow$', 'Interpreter', 'latex', 'FontSize', 14)

ylabel('$V \rightarrow$', 'Interpreter', 'latex', 'FontSize', 14)

legend({'NRZ(VN)'}, 'Interpreter', 'latex', 'FontSize', 14);

## B 1.2 Determine and plot the Continuous Time (CT) signal for the entire string:

Transmission Rate = 1 Mbps=106bps

Bit Duration = 1Transmission Rate=10-6s

Number of cycles in a bit 0 with 10 Mhz frequency,

=bit duration×frequency
=10-6×10×106
=10 Cycles

Number of cycles in a bit 1 with 5 Mhz frequency,

=bit duration×frequency
=10-6×5×106
=5 Cycles

freq_zero = 10*10^6;

freq_one = 5*10^6;

bit_duration = 10^-6;

bits = [1 0 1 0 1 1 0 1 0 0 1 1 1 0];

A = 3.5;

time = 0:bit_duration:length(bits)*bit_duration;

syms t;

for i = 1:length(bits)

if bits(i) == 0

f = freq_zero;

else

f = freq_one;

end

fplot(A*sin(2*pi*f*t), [time(i) time(i+1)], 'color', [.5 .4 .7], 'LineWidth', 2);

hold on;

end

axis([0 max(time) -5 5])

xticks(time)

yticks([-3.5 0 3.5])

ax1 = gca;

set(ax1, 'FontSize', 14);

grid on;

xlabel('$t \rightarrow$', 'Interpreter', 'latex', 'FontSize', 14)

ylabel('$V \rightarrow$', 'Interpreter', 'latex', 'FontSize', 14)

legend({'SINE PULSE NRZ(VN)'}, 'Interpreter', 'latex', 'FontSize', 14);

## B 1.3 Compute the energy for the characters in the text and energy per character of the CT Signal

The Energy of the Non-Periodic signal obtained in B1.2 is given by

E=-∞∞Asinωt2dt

Where,

A:amplitude of the signal
ω:2πf angular frequency
t:time

E= A2-∞∞sin22πftdt
=A2-∞∞12-cos4πft2dt
=A22-∞∞dt-A22sin4πft4πf-∞∞[sin2nπ=0]
=A22-∞∞dt

Now we can compute it for the entire signal,

E=A2201×10-6dt+1×10-62×10-6dt+2×10-63×10-6dt+3×10-64×10-6dt+4×10-65×10-6dt+5×10-66×10-6dt+6×10-67×10-6dt+7×10-68×10-6dt+8×10-69×10-6dt+9×10-610×10-6dt+10×10-611×10-6dt+11×10-612×10-6dt+12×10-613×10-6dt+13×10-614×10-6dt
E=3.522×14×10-6
E=85.75×10-6J

Since there are 2 characters, Energy per character is,

Echaracter=85.752×10-6
Echaracter=42.875×10-6J

## B 1.4 Conclusion

Students are expected to draw conclusions based on the discussions and suggestions (not to exceed 100 words)

# Question No. 3

Solution to Question No. 2 Part B:

Given data:

xt  : Amplitude 6, from t = -1 to t = 1

h1(t): Amplitude 2, from t = 0 to t = 2

h2(t): Amplitude 4, from t = 2 to t = 3

Plots:

## B 2.1 Compute and plot the signal received by the second link:

The given signal x(t) is in terms of the variable t, which is first converted to the variable τ, to give,

xτ:

Now to get h1(t-τ), first the signal h1(t) is converted to h1(τ) and then reflection of it is done to get h1(-τ), to this formed signal we add t to get our final signal,

h1t-τ:

Comparing the values of overlaps from the x-axis we obtain,

| t≤-1 | t≤-1 | t≤-1 -1≤t≤1 1≤t≤3 t≥3 |
| --- | --- | --- |
| t≥-1 | t≥-1 |  |
| t≤1 | t≤1 |  |
| t≥1 | t≥1 |  |
| t-2≤-1 | t≤1 |  |
| t-2≥-1 | t≥1 |  |
| t-2≤1 | t≤3 |  |
| t-2≥1 | t≥3 |  |

CASE 1

| Range : t≤-1 Terms involved : t≤-1 y1t=0 |  |
| --- | --- |

CASE 2

| Range : -1≤t≤1 Terms involved : t≥-1, t≤1, t-2≤-1 y1t=-1t26dτ y1t=12t+12 |  |
| --- | --- |

CASE 3

| Range : 1≤t≤3 Terms involved : t≥1, t-2≥-1, t-2≤1 y1t=t-2t26dτ y1t=36-12t |  |
| --- | --- |

CASE 4

| Range : t≥3 Terms involved : t-2≥1 y1t=0 |  |
| --- | --- |

Hence the output from the first convolution is:

y1t=12t+12×ut+1-ut-1+36-12t×(ut-1-u(t-3))

%% plot for y1(t)

syms t

hold off;

y1 = (12*t+12).*(heaviside(t+1)-heaviside(t-1)) + (36-12*t).*(heaviside(t-1)-heaviside(t-3));

fplot(y1, [-3 4], 'LineWidth', 2)

xlabel('$\tau \rightarrow$', 'Interpreter', 'latex', 'FontSize', 14)

legend({'$y_1(\tau)$'}, 'Interpreter', 'latex', 'FontSize', 14);

grid on;

ax1 = gca;

set(ax1, 'FontSize', 14);

axis([-3 4 -1 26])

yticks([0 4 10 24])

## B 2.2 Compute and plot the signal received at the destination:

The output from the previous link is the input for the second link which will give the signal received at the destination,

y1τ:

Now to get h2(t-τ), first the signal h2(t) is converted to h2(τ) and then reflection of it is done to get h2(-τ), to this formed signal we add t to get our final signal,

h2t-τ:

Comparing the values of overlaps from the x-axis we obtain,

| t-2≤-1 | t≤1 | t≤1 1≤t≤2 2≤t≤3 3≤t≤4 4≤t≤5 5≤t≤6 t≥6 |
| --- | --- | --- |
| t-2≥-1 | t≥1 |  |
| t-2≤1 | t≤3 |  |
| t-2≥1 | t≥3 |  |
| t-2≤3 | t≤5 |  |
| t-2≥3 | t≥5 |  |
| t-3≤-1 | t≤2 |  |
| t-3≥-1 | t≥2 |  |
| t-3≤1 | t≤4 |  |
| t-3≥1 | t≥4 |  |
| t-3≤3 | t≤6 |  |
| t-3≥3 | t≥6 |  |

CASE 1

| Range : t≤1 Terms involved : t-2≤-1 yt=0 |  |
| --- | --- |

CASE 2

| Range : 1≤t≤2 Terms involved : t-2≥-1, t-3≤-1 yt=-1t-212τ+124dτ yt=24t-12 |  |
| --- | --- |

CASE 3

| Range : 2≤t≤3 Terms involved : t-3≥-1, t-2≤1 yt=t-3t-212τ+124dτ yt=48t-72 |  |
| --- | --- |

CASE 4

| Range : 3≤t≤4 Terms involved : t-2≥1, t-3≤1 yt=t-3112τ+124dτ+ 1t-236-12τ4dτ yt=-24t-3t-7-24tt-4 yt=-48t2+336t-504 |  |
| --- | --- |

CASE 5

| Range : 4≤t≤5 Terms involved : t-3≤1, t-2≤3 yt=t-3t-236-12τ4dτ yt=264-48t |  |
| --- | --- |

CASE 6

| Range : 5≤t≤6 Terms involved : t-2≥3, t-3≤3 yt=t-3336-12τ4dτ yt=24t-62 |  |
| --- | --- |

CASE 7

| Range : t≥6 Terms involved : t-3≥3 yt=0 |  |
| --- | --- |

Hence the output from the second convolution is:

yt= 24t-12;1≤t≤2 48t-72;2≤t≤3-48t2+336t-504;3≤t≤4264-48t;4≤t≤524t-62;5≤t≤60; otherwise

%% plot for y(t)

syms t;

y = (24*(t-1).^2).*(heaviside(t-1)-heaviside(t-2)) + ...

(48*t-72).*(heaviside(t-2)-heaviside(t-3)) + ...

(-48*t.^2+336*t-504).*(heaviside(t-3)-heaviside(t-4)) + ...

(264-48*t).*(heaviside(t-4)-heaviside(t-5)) + ...

(24*(t-6).^2).*(heaviside(t-5)-heaviside(t-6));

fplot(y , [-1 8], 'LineWidth', 2);

axis([-1 8 -5 90]);

grid on;

xlabel('$t \rightarrow$', 'Interpreter', 'latex', 'FontSize', 14)

ax1 = gca;

set(ax1, 'FontSize', 14);

legend({'$y(t)$'}, 'Interpreter', 'latex', 'FontSize', 14);

## B 2.3 Obtain the formula for the overall impulse response of the network:

The impulse response at first link:

y1t=-∞∞xτh1t-τdτ
yt=-∞∞y1σh2t-σdσ

Which can be written as,

yt=-∞∞-∞∞xτh1t-τdτh2t-τdτ

Bibliography

________________________________________________________________________________